# INT303: Networking Fundamentals

# Lab 5: IP Address Analysis and Network Report

Student: Damian Patrick Ozimede

Course: INT303 — Networking Fundamentals

Lab: Lab 5

Date: June 2026

## Introduction

This lab analyses 10 real-world websites to understand IP addressing, classification, subnetting, and network scalability. By pinging each website, recording their IP addresses, classifying them by class (A, B, or C), calculating device capacity, and determining subnet masks, this report demonstrates practical application of IP networking concepts. Advanced tools such as traceroute and whois were also used to investigate routing paths and verify the ownership of IP addresses through official registry data.

## Website List and IP Addresses

| Website | IP Address | Ping Response |
| --- | --- | --- |
| google.com | 142.251.216.110 | No reply (ICMP blocked) |
| apple.com | 17.253.144.10 | 180 ms |
| microsoft.com | 150.171.109.2 | No reply (ICMP blocked) |
| amazon.com | 98.87.170.74 | No reply (ICMP blocked) |
| facebook.com | 57.144.38.1 | 58.8 ms |
| twitter.com | 162.159.140.229 | 176 ms |
| wikipedia.org | 185.15.58.224 | 207 ms |
| github.com | 140.82.121.3 | 160 ms |
| netflix.com | 54.155.178.5 | No reply (ICMP blocked) |
| bbc.co.uk | 151.101.128.81 | 205 ms |

### Bonus: Multiple IP Addresses (CDN/Load Balancing)

The dig command was used to check for multiple IP addresses per domain:

google.com — 1 IP only
amazon.com — 3 IPs: 98.87.170.71, 98.87.170.74, 98.82.161.185
facebook.com — 1 IP only
netflix.com — 3 IPs: 54.155.246.232, 18.200.8.190, 54.73.148.110
microsoft.com — 1 IP only

Amazon and Netflix clearly use load balancing — multiple servers handle traffic simultaneously across different geographic regions.

## IP Address Classification

IP addresses are classified based on their first octet:
Class A: 1-126 (default mask /8)
Class B: 128-191 (default mask /16)
Class C: 192-223 (default mask /24)

Only the first number of the IP address is needed to determine the class.

| Website | IP Address | First Octet | Class |
| --- | --- | --- | --- |
| google.com | 142.251.216.110 | 142 | B |
| apple.com | 17.253.144.10 | 17 | A |
| microsoft.com | 150.171.109.2 | 150 | B |
| amazon.com | 98.87.170.74 | 98 | A |
| facebook.com | 57.144.38.1 | 57 | A |
| twitter.com | 162.159.140.229 | 162 | B |
| wikipedia.org | 185.15.58.224 | 185 | B |
| github.com | 140.82.121.3 | 140 | B |
| netflix.com | 54.155.178.5 | 54 | A |
| bbc.co.uk | 151.101.128.81 | 151 | B |

### Bonus: Reserved IP Ranges

All IP addresses fall into one of two categories:

Public IPs — Unique addresses used on the internet. All 10 websites use public IPs.

Private IPs — Reserved ranges used only inside private networks, never directly on the internet:
10.0.0.0 - 10.255.255.255 (10.0.0.0/8) — Class A private range
172.16.0.0 - 172.31.255.255 (172.16.0.0/12) — Class B private range
192.168.0.0 - 192.168.255.255 (192.168.0.0/16) — Class C private range

None of the 10 IP addresses fall into these reserved private ranges. All are public IPs assigned to large organisations.

Significance: Private IP ranges allow organisations to use the same internal addresses (e.g. 192.168.x.x) without conflicts, since these are never routed on the public internet. Routers use NAT (Network Address Translation) to translate private IPs to a single public IP when accessing the internet, conserving the limited pool of public IPv4 addresses.

## Number of Devices Supported

Device capacity is calculated based on the number of bits remaining for host addresses after the network portion:
Class A: 32-8 = 24 host bits = 2^24 = 16,777,216 addresses
Class B: 32-16 = 16 host bits = 2^16 = 65,536 addresses
Class C: 32-24 = 8 host bits = 2^8 = 256 addresses

Usable hosts = Total addresses - 2 (network address and broadcast address are reserved and cannot be assigned to devices).

| Website | Class | Total Addresses | Usable Hosts |
| --- | --- | --- | --- |
| google.com | B | 65,536 | 65,534 |
| apple.com | A | 16,777,216 | 16,777,214 |
| microsoft.com | B | 65,536 | 65,534 |
| amazon.com | A | 16,777,216 | 16,777,214 |
| facebook.com | A | 16,777,216 | 16,777,214 |
| twitter.com | B | 65,536 | 65,534 |
| wikipedia.org | B | 65,536 | 65,534 |
| github.com | B | 65,536 | 65,534 |
| netflix.com | A | 16,777,216 | 16,777,214 |
| bbc.co.uk | B | 65,536 | 65,534 |

### Bonus: Subnetting a Class B Network with /26

Taking google.com (142.251.216.110) as an example and subnetting with /26:
/26 subnet mask = 255.255.255.192
Class B default = /16, so /26 borrows 26-16 = 10 extra bits for subnets
Number of subnets = 2^10 = 1,024 subnets
Hosts per subnet = 2^(32-26) - 2 = 2^6 - 2 = 62 usable hosts

This shows how subnetting a large Class B network into /26 subnets creates 1,024 smaller segments each supporting 62 devices — useful for dividing large organisations into smaller manageable network segments such as departments or offices.

This calculation can also be applied to Class A. If a Class A network (/8) is subnetted to /26, it would create 2^(26-8) = 2^18 = 262,144 subnets, each still supporting 62 hosts. The hosts-per-subnet figure depends only on the final CIDR value, while the number of subnets depends on how many bits were borrowed from the original class.

## Subnet Masks

| Website | Class | Default Subnet Mask | CIDR |
| --- | --- | --- | --- |
| google.com | B | 255.255.0.0 | /16 |
| apple.com | A | 255.0.0.0 | /8 |
| microsoft.com | B | 255.255.0.0 | /16 |
| amazon.com | A | 255.0.0.0 | /8 |
| facebook.com | A | 255.0.0.0 | /8 |
| twitter.com | B | 255.255.0.0 | /16 |
| wikipedia.org | B | 255.255.0.0 | /16 |
| github.com | B | 255.255.0.0 | /16 |
| netflix.com | A | 255.0.0.0 | /8 |
| bbc.co.uk | B | 255.255.0.0 | /16 |

### Bonus: Custom Subnetting Scenarios (/25, /26, /27)

Using a Class B network (e.g. google.com 142.251.216.0) with custom subnet masks:

| Subnet Mask | CIDR | Hosts per Subnet | Number of Subnets |
| --- | --- | --- | --- |
| 255.255.255.128 | /25 | 126 | 512 |
| 255.255.255.192 | /26 | 62 | 1,024 |
| 255.255.255.224 | /27 | 30 | 2,048 |

How the subnet mask values were derived:
Subnet masks are built in binary, with each bit position in the final octet worth: 128, 64, 32, 16, 8, 4, 2, 1
/25 turns on 1 bit beyond /24 = 128
/26 turns on 2 bits = 128+64 = 192
/27 turns on 3 bits = 128+64+32 = 224

Hosts per subnet formula: 2^(32-CIDR) - 2
Number of subnets formula (from Class B /16 base): 2^(CIDR-16)

### Custom Subnetting Scenarios: Discussion

Scenarios where custom subnetting is required: A large organisation with a single Class B network (65,534 hosts) rarely puts every device on one flat network. Instead they divide it by department or location — for example separate subnets for Finance, HR, IT, and Sales — each needing only 50-100 devices.

How altering the subnet mask affects device capacity and organization: Moving from /25 to /27, device capacity decreases (126 to 30 hosts) while the number of subnets increases (512 to 2,048). This is a direct tradeoff — borrowing more bits for subnets leaves fewer bits for hosts.

Benefits of smaller subnets: Better security (a breach in one subnet does not directly expose others), reduced broadcast traffic (broadcasts stay within each smaller subnet improving performance), and easier troubleshooting (issues isolated to specific subnets).

Drawback: More subnets means more complex routing tables and IP address management.

This mirrors the approach used in Lab 4 where separate subnets (192.168.10.0/24 and 192.168.20.0/24) were created with iptables controlling traffic between them — simulating real organisational network segmentation.

## Advanced Tools and Analysis

### Traceroute to apple.com

Command used: traceroute apple.com

Results:
Hop 1 — 192.168.5.2 (5.1-5.3 ms) — Local gateway, a private IP address
Hops 2-30 — All * * * — No response from ISP/backbone routers

Analysis: Hop 1 is the local network gateway, a private IP (192.168.0.0/16 range), confirming the Kali machine exits the local network through this gateway with excellent response time (~5ms).

Hops 2-30 returned * * *, meaning intermediate routers did not respond to traceroute probes. This is normal behaviour — ISPs and backbone providers commonly drop ICMP/UDP traceroute probes for security and privacy reasons, preventing network topology mapping.

Despite this, the ping to apple.com (17.253.144.10) succeeded with a response time of 180 ms, confirming traffic does reach Apple's servers successfully — the intermediate hops are simply invisible to traceroute, not actually broken.

Significance: This demonstrates that traceroute only shows hops where devices choose to respond. The actual path likely passes through multiple ISP routers and internet exchange points, but these are invisible due to security configurations.

Private vs infrastructure: Hop 1 (192.168.5.2) is a private network device (local router). Apple's destination IP (17.253.144.10) belongs to Apple's own infrastructure rather than a third-party cloud provider.

### GeoIP Analysis (Verified via WHOIS)

The whois command was used to query the ARIN (American Registry for Internet Numbers) database for official documented proof of IP address ownership:

apple.com (17.253.144.10):
NetRange: 17.0.0.0 - 17.255.255.255
CIDR: 17.0.0.0/8
Organization: Apple Inc. (APPLEC-1-Z)
This confirms Apple owns the entire 17.0.0.0/8 address block — an extremely rare allocation size reserved only for very large organisations operating their own global private network.

github.com (140.82.121.3):
NetRange: 140.82.112.0 - 140.82.127.255
CIDR: 140.82.112.0/20
Organization: GitHub, Inc. (GITHU)
This confirms GitHub directly owns this address block, consistent with the hostname "lb-140-82-121-3-fra" showing a load balancer in Frankfurt, Germany.

bbc.co.uk (151.101.128.81):
NetRange: 151.101.0.0 - 151.101.255.255
CIDR: 151.101.0.0/16
Organization: Fastly, Inc. (SKYCA-3)
This confirms BBC's website is served through Fastly's CDN infrastructure rather than BBC's own servers.

### Data Centers, ISPs, or Specific Regions

Belonging to large tech company data centers (owned infrastructure):
apple.com (17.253.144.10) — Apple owns its own IP blocks and operates its own global data centers
facebook.com (57.144.38.1) — Hostname "edge-star-mini-shv-01-los4" reveals Meta's edge server in Los Angeles
github.com (140.82.121.3) — Hostname "lb-140-82-121-3-fra" shows GitHub's load balancer in Frankfurt, Germany
wikipedia.org (185.15.58.224) — Hostname "text-lb.drmrs.wikimedia.org" shows Wikimedia's server in Marseille, France (DRMRS = Marseille airport code)

Belonging to cloud provider infrastructure (rented data centers):
amazon.com (98.87.170.74) and netflix.com (54.155.178.5, 18.200.8.190, 54.73.148.110) — These IPs fall within Amazon AWS address ranges, indicating both run on AWS cloud infrastructure across multiple regions

Belonging to CDN providers:
bbc.co.uk (151.101.128.81) — Belongs to Fastly, a CDN provider BBC uses to distribute content closer to users
twitter.com (162.159.140.229) — Belongs to Cloudflare, used for CDN and DDoS protection

Belonging to standard global network backbones:
google.com (142.251.216.110) and microsoft.com (150.171.109.2) — These belong to Google's and Microsoft's own global network backbones, distributing traffic to regional data centers

In summary, all 10 websites are hosted on large-scale infrastructure — either owned directly by the company, rented from major cloud providers, or distributed through CDNs. This reflects how major websites rely on globally distributed infrastructure for fast and reliable access worldwide.

### Latency vs Geographic Distance

| Website | Response Time | Likely Location |
| --- | --- | --- |
| facebook.com | 58.8 ms | Los Angeles, USA (local edge caching likely) |
| github.com | 160 ms | Frankfurt, Germany |
| twitter.com | 176 ms | Cloudflare CDN (European edge) |
| apple.com | 180 ms | USA |
| bbc.co.uk | 205 ms | Fastly CDN (UK/Europe) |
| wikipedia.org | 207 ms | Marseille, France |

Observation: Facebook's significantly lower latency (58.8 ms) despite being hosted in Los Angeles suggests Meta operates a local caching/edge server within Nigeria or West Africa, while other services rely on more distant European or US servers, resulting in higher latencies of 160-207 ms.

## Conclusion

This lab provided practical experience analysing real-world IP addresses across 10 major websites. By classifying each IP into Class A or B, calculating device capacities ranging from 65,534 to over 16 million hosts, and exploring subnet mask variations, this report demonstrates how IP address management directly impacts network scalability and organisation.

The traceroute exercise reinforced findings from earlier labs — ISPs commonly block traceroute probes for security reasons, though traffic still flows successfully. The whois lookups provided documented proof of IP ownership, confirming that major websites rely on a mix of owned infrastructure (Apple, GitHub, Wikimedia), cloud providers (AWS for Amazon and Netflix), and CDNs (Fastly for BBC, Cloudflare for Twitter).

Understanding IP classification, subnetting, and address allocation is fundamental to network configuration, security auditing, and system administration. The custom subnetting exercise (/25, /26, /27) demonstrated the direct tradeoff between subnet size and the number of available subnets — a critical consideration when designing scalable and secure network architectures for real organisations.